DoDo Bot — Perception Stack

Week 1, Day 1: Data Collection Setup (Obstacle Detection)

# Objective

Set up synchronized recording of LiDAR and OAK-D RGB-D streams in a restaurant-like environment (tables, chair legs, moving people, trays/trolleys) so this data can later be labeled and used to train the obstacle detection models (Days 2–4).

# Deliverables for Today

- ROS2 workspace with a data_collection package

- Launch file that brings up LiDAR + OAK-D drivers together

- A recording node/script using rosbag2 to capture synchronized topics

- 2–3 recorded bag files covering different obstacle scenarios

- Quick sanity check: playback + rviz2 visualization

# 1. Workspace Setup

Create the ROS2 package that will hold our data collection tools.

mkdir -p ~/dodo_ws/src
cd ~/dodo_ws/src
ros2 pkg create --build-type ament_python data_collection \
  --dependencies rclpy sensor_msgs geometry_msgs message_filters
cd ~/dodo_ws
colcon build --packages-select data_collection
source install/setup.bash

# 2. Launch File — Bring Up Sensors Together

Path: ~/dodo_ws/src/data_collection/launch/sensors.launch.py

from launch import LaunchDescription
from launch_ros.actions import Node

      
def generate_launch_description():
    return LaunchDescription([
        # LiDAR driver (adjust for your specific LiDAR, e.g. RPLIDAR/Livox)
        Node(
            package='rplidar_ros',
            executable='rplidar_node',
            name='rplidar_node',
            parameters=[{
                'serial_port': '/dev/ttyUSB0',
                'frame_id': 'laser_frame',
                'angle_compensate': True
            }],
            output='screen'
        ),
        # OAK-D driver (depthai-ros)
        Node(
            package='depthai_ros_driver',
            executable='camera_node',
            name='oakd_camera',
            parameters=[{
                'camera.i_publish_topic': True,
                'rgb.i_publish_topic': True,
                'stereo.i_publish_topic': True
            }],
            output='screen'
        ),
    ])

# launch it
ros2 launch data_collection sensors.launch.py

# 3. Synchronized Recorder Node

This node uses message_filters to time-align LiDAR scans with RGB + depth frames before saving, which avoids mismatched pairs later during labeling. Path: ~/dodo_ws/src/data_collection/data_collection/sync_recorder.py

import rclpy
from rclpy.node import Node
from message_filters import ApproximateTimeSynchronizer, Subscriber
from sensor_msgs.msg import LaserScan, Image

      
class SyncRecorder(Node):
    def __init__(self):
        super().__init__('sync_recorder')

      
        self.lidar_sub = Subscriber(self, LaserScan, '/scan')
        self.rgb_sub = Subscriber(self, Image, '/oakd/rgb/image_raw')
        self.depth_sub = Subscriber(self, Image, '/oakd/stereo/image_raw')

      
        self.ts = ApproximateTimeSynchronizer(
            [self.lidar_sub, self.rgb_sub, self.depth_sub],
            queue_size=10,
            slop=0.05  # 50ms tolerance across sensors
        )
        self.ts.registerCallback(self.callback)
        self.frame_count = 0
        self.get_logger().info('Sync recorder ready, waiting for frames...')

      
    def callback(self, scan_msg, rgb_msg, depth_msg):
        self.frame_count += 1
        if self.frame_count % 30 == 0:
            self.get_logger().info(f'Synced frames captured: {self.frame_count}')
        # Actual saving to disk is handled by rosbag2 (see Section 4).
        # This node exists mainly to verify sync quality in real time.

      
def main(args=None):
    rclpy.init(args=args)
    node = SyncRecorder()
    rclpy.spin(node)
    node.destroy_node()
    rclpy.shutdown()

      
if __name__ == '__main__':
    main()

# 4. Recording with rosbag2

Run this alongside the sync_recorder above. Name each bag after the scenario being captured (e.g. static_chairs, moving_people, mixed_clutter).

# Record only the topics we need (keeps bag size manageable)
ros2 bag record -o dodo_scenario_static_chairs \
  /scan \
  /oakd/rgb/image_raw \
  /oakd/stereo/image_raw \
  /oakd/rgb/camera_info \
  /tf /tf_static

      
# Repeat for each scenario, e.g.:
ros2 bag record -o dodo_scenario_moving_people /scan /oakd/rgb/image_raw \
  /oakd/stereo/image_raw /oakd/rgb/camera_info /tf /tf_static

# 5. Sanity Check — Playback + Visualization

# Playback
ros2 bag play dodo_scenario_static_chairs

      
# In a separate terminal, visualize
rviz2
# Add displays: LaserScan (/scan), Image (/oakd/rgb/image_raw),
# Image (/oakd/stereo/image_raw), TF

Check that the LiDAR scan and camera frames stay visually aligned as a person or chair moves through frame — drift here means the slop value or driver timestamps need adjusting before you commit to further recording.

# Alternative Track: Bootstrap with a Public Dataset

If the LiDAR + OAK-D recording setup above isn't ready yet, or the team wants to see model results sooner, Day 1 can instead use an existing dataset to get the pipeline running end-to-end. This is a bootstrap, not a replacement — the model will still need fine-tuning on real DoDo Bot recordings later (Days 2–4) since camera angle, mount height, and floor layout won't match a public dataset.

## Datasets to Use

| Dataset | Why it fits |
| --- | --- |
| COCO | General object classes (person, chair, dining table, backpack) — good base for fine-tuning YOLOv8 |
| SUN RGB-D / NYU Depth V2 | Indoor scenes with depth data, closer to the restaurant setting |
| OpenImages (filtered) | Larger volume of chair/table/person images if COCO feels limited |
| KITTI / TUM RGB-D | Lower priority — built for outdoor/SLAM benchmarking, not indoor restaurant scenes |

## Quick Baseline: Download + Fine-tune YOLOv8 on COCO subset

# In VS Code / local Python env (NOT ROS2)
pip install ultralytics fiftyone

      
# Pull a filtered COCO subset with just the classes we care about
python3 - <<'EOF'
import fiftyone as fo
import fiftyone.zoo as foz

      
dataset = foz.load_zoo_dataset(
    'coco-2017',
    split='train',
    label_types=['detections'],
    classes=['person', 'chair', 'dining table', 'backpack', 'handbag'],
    max_samples=2000,
)
dataset.export(
    export_dir='./coco_subset_yolo',
    dataset_type=fo.types.YOLOv5Dataset,
)
EOF

# Fine-tune YOLOv8 baseline on the exported subset
yolo detect train \
  data=./coco_subset_yolo/dataset.yaml \
  model=yolov8n.pt \
  epochs=30 \
  imgsz=640 \
  name=dodo_obstacle_baseline

This gets a working detection pipeline (data, training, weights) running today, so the pipeline itself is already proven by the time real DoDo Bot recordings are ready to swap in.

## Recommended Approach for Today

- Primary: Download COCO/SUN RGB-D subset and run the YOLOv8 baseline fine-tune above

- In parallel or tomorrow: still capture 10-15 minutes of real ROS2 footage from the actual restaurant setup (Sections 1-5 above), even if brief

- Compare baseline performance on public data vs. a few real DoDo Bot frames early, to catch domain gap before Week 3 rather than discovering it late

# 6. Suggested Scenarios to Capture Today (If Recording Live)

| Scenario | What it covers |
| --- | --- |
| static_chairs | Chair/table legs at various distances — the trickiest low-profile obstacles for LiDAR |
| moving_people | 1–3 people walking across the robot's path at different speeds |
| mixed_clutter | Trays, trolleys, bags left on the floor combined with people |
| narrow_passage | Tight aisles between tables to test close-range detection |

# Where Training Happens (Not Today, But Noting the Split)

ROS2 is only used for capturing and later deploying data — not for training. Once today's bags are recorded, extraction and labeling happen outside ROS2 in a normal Python/VS Code environment:

- ROS2 (today): drivers, synchronized recording, rosbag2 storage

- VS Code + Python env (Day 2 onward): extract frames/scans from bags, label, train YOLOv8/clustering models

- Back to ROS2 (Day 5): trained model weights are loaded into a ROS2 inference node for live obstacle avoidance

# Example: extracting images from a bag for offline training later
ros2 run rosbag2_tools bag_to_images \
  --bag-path dodo_scenario_static_chairs \
  --topic /oakd/rgb/image_raw \
  --output-dir ~/dodo_ws/extracted/static_chairs

# End-of-Day Checklist

Live Recording Track:

- [ ] Workspace builds cleanly with colcon build

- [ ] LiDAR + OAK-D publish topics simultaneously

- [ ] sync_recorder confirms frames are time-aligned (slop < 50ms)

- [ ] At least 3 scenario bags recorded

- [ ] Bags verified via playback + rviz2, no dropped/misaligned frames

Dataset Bootstrap Track:

- [ ] COCO/SUN RGB-D subset downloaded and filtered to relevant classes

- [ ] YOLOv8 baseline fine-tuned end-to-end (even a few epochs counts)

- [ ] At least a few real DoDo Bot frames captured for early domain-gap comparison